AgriRover ROS2 System

Safety Mechanisms & Test Plan

Date: March 20, 2026

Dual autonomous ground rover platform — precision agriculture

# 1.  System Overview

The AgriRover system consists of two autonomous ground rovers (RV1 master, RV2 slave), each driven by a Jetson Orin Nano running ROS2. Safety is implemented in four independent layers so that a failure in any single layer still brings the rovers to a safe neutral state.

Rover hardware:

RV1 (master) — Jetson Orin Nano, HM30 air unit, RP2040 with SBUS → SX1278 TX

RV2 (slave)  — Jetson Orin Nano, RP2040 with SX1278 RX → PPM output

GQC — SIYI MK32 Android ground control station, MAVLink over WiFi (sysid 255)

Safety layers (innermost to outermost):

Layer 1 — RP2040 firmware  (hardware PPM at 50 Hz, fully independent of Jetson)

Layer 2 — ROS2 nodes on Jetson  (navigator, rp2040_bridge, mavlink_bridge)

Layer 3 — Android GQC application  (operator alerts and interlocks)

Layer 4 — Physical RC hardware  (transmitter switch, RF link integrity)

# 2.  Safety Mechanism Inventory

## 2.1  Layer 1 — RP2040 Firmware (Hardware PPM)

These mechanisms run inside the RP2040 microcontroller at 50 Hz, entirely independent of the Jetson and ROS2.  All result in PPM channels returning to neutral (1500 µs) unless otherwise stated.

| # | Mechanism | Rover | Trigger | Effect |
| --- | --- | --- | --- | --- |
| 1 | SBUS signal timeout | Master (RV1) | No SBUS frame for 200 ms (transmitter off or HM30 link lost) | MODE_EMERGENCY → all PPM → 1500 µs Logs [SBUS_LOST] |
| 2 | Emergency switch SWA (CH4) | Both rovers | sbus_ch[4] or rf_ch[4] < 1700 µs (physical 2-position switch flipped) | MODE_EMERGENCY → all PPM → 1500 µs on both rovers simultaneously |
| 3 | Jetson heartbeat timeout | Both rovers | No <HB:N> from rp2040_bridge for 300 ms | hb_alive = false → AUTO blocked MODE_AUTO_NO_HB → PPM neutral |
| 4 | Autonomous command timeout | Both rovers | No <J:c0..c7> from navigator for 500 ms | cmd_fresh = false → MODE_AUTO_TIMEOUT → PPM neutral (rover coasts to stop) |
| 5 | RF link timeout | Slave (RV2) | No LoRa packet for 500 ms | MODE_EMERGENCY → all PPM → 1500 µs Logs [RF_LINK_LOST] |
| 6 | SX1278 power-cycle watchdog | Slave (RV2) | RF lost + RegOpMode ≠ 0x85 every 2 s | Automatic chip reinit + RX restart Logs [SX1278_REINIT opmode=0x...] |
| 7 | PPM speed limit | Both rovers | Always active — all modes (MANUAL, AUTONOMOUS, RF relay) | Throttle (CH1, PPM idx 0) and spin (CH2, PPM idx 1) hard-clamped to [1350–1650 µs] = ±30% of range. MK32 shows full range; hardware PPM capped. CH3–CH8 (switches, servos) unaffected. |

## 2.2  Layer 2 — ROS2 Nodes (Jetson)

These run on each Jetson Orin Nano. They depend on the RP2040 being responsive but are independent of GQC connectivity.

| # | Mechanism | Node | Trigger | Effect |
| --- | --- | --- | --- | --- |
| 8 | Heartbeat to RP2040 | rp2040_bridge | Continuous 10 Hz loop | Sends <HB:N> to keep hb_alive on RP2040. Stopping this node triggers mechanism #3 within 300 ms. |
| 9 | armed topic gate | navigator | armed topic = false | Navigator stops publishing cmd_override. RP2040 reaches AUTO_TIMEOUT within 500 ms. |
| 10 | mode topic gate | navigator | mode topic ≠ "AUTONOMOUS" | Navigator does not compute or publish motion commands. |
| 11 | RTK fix-loss safety | navigator | GPS fix type drops below RTK threshold | Autonomous motion stopped. Mission recording disabled. GQC RTK badge updated. |
| 12 | Mission-complete auto-disarm | mavlink_bridge | wp_active = −1 published + no pending chunks | Publishes MANUAL mode + DISARM. STATUS badge drops to NA. Fires only at true mission end. |

## 2.3  Layer 3 — Android GQC Application

Operator-facing interlocks and alerts. Critical commands use 3× UDP retry (100 ms spacing) to tolerate packet loss.

| # | Mechanism | Trigger | Effect |
| --- | --- | --- | --- |
| 13 | E-STOP button | Operator tap at any time | DISARM (MAVCmd 400, p1=0) × 3 at 100 ms intervals to broadcast. Reaches both rovers simultaneously regardless of selection. |
| 14 | DISARM before mission upload | UPLOAD button pressed | Sends DISARM × 3 before MISSION_COUNT. Prevents accidental motion during mission load. |
| 15 | ARM safety gate (START button) | START pressed in AUTO mode | Blocks ARM + shows alert if: (a) No mission loaded — STATUS = NA (b) Rover STATUS = NA on rover side (c) Rover GPS > 0.5 m from WP[0] |
| 16 | RC/HB link-mismatch auto-disarm | CH9 switch ≠ slave HEARTBEAT for > 2 s | checkLinkMismatch() detects inconsistency. Sends DISARM × 3 automatically. Logs mismatch event. |
| 17 | HUD visual alerts | Continuous telemetry monitoring | SBUS dot (red=lost), RF dot (red=lost), RTK badge (colour=fix quality), STATUS badge (NA/MSL/ARM), HB dot blink. All initialised RED — require positive confirm to show green. |

## 2.4  Layer 4 — Physical RC Hardware

| # | Mechanism | Trigger | Effect |
| --- | --- | --- | --- |
| 18 | Transmitter signal loss | HM30 RF link drops or transmitter powered off | SBUS signal lost on master RP2040 → mechanism #1 fires within 200 ms. |
| 19 | SWA emergency switch (CH4) | Operator flips 2-position switch on MK32 | Mechanism #2 fires simultaneously on both rovers via RP2040 firmware. Does not require Jetson or GQC connectivity. |

# 3.  Safety Test Plan

Each test is self-contained. "Pass condition" describes the observable outcome confirming correct behaviour. Tests are ordered from safest (bench / no motion) to full mission runs. Complete T1–T7 before the first outdoor drive.

Notation: "serial log" = USB CDC output from RP2040, readable with any serial terminal at 115200 baud or from monitor.py dashboard.

## T1 — SBUS Signal Timeout  [Mechanism #1]

| Field | Details |
| --- | --- |
| Prerequisites | Master RP2040 connected via USB. Rover on bench. Transmitter on, SBUS OK. |
| Setup | Enable MANUAL mode. Apply slight throttle so PPM CH1 ≠ 1500. |
| Action | Power off the MK32 transmitter (or disconnect HM30 cable). |
| Observe | RP2040 serial log shows [SBUS_LOST] and MODE:EMERGENCY within 200 ms. |
| Pass condition | CH1 and CH2 PPM return to 1500 µs within 200 ms. Mode = EMERGENCY. |
| Restore | Power transmitter back on; SBUS_OK resumes automatically. |

## T2 — Emergency Switch SWA  [Mechanism #2]

| Field | Details |
| --- | --- |
| Prerequisites | Both rovers powered. Transmitter on with SBUS OK. |
| Setup | MANUAL mode, slight motion on one rover. |
| Action | Flip SWA (CH4) to the emergency/low position (< 1700 µs). |
| Observe | Both rover serial logs show MODE:EMERGENCY. PPM → 1500 immediately. |
| Pass condition | Both rovers stop while transmitter is still on and SBUS link is healthy. |
| Restore | Flip SWA back to safe (high) position. |

## T3 — Jetson Heartbeat Timeout  [Mechanism #3]

| Field | Details |
| --- | --- |
| Prerequisites | Rover running full ROS2 stack. Armed in AUTO mode. |
| Setup | Navigator running and sending motion commands. |
| Action | Kill the rp2040_bridge node only (Ctrl-C or ros2 lifecycle shutdown). |
| Observe | Within 300 ms: serial log shows AUTO-NO-HB; PPM returns to neutral. |
| Pass condition | Rover stops within 300 ms. Navigator remains running and resumes when bridge restarts. |
| Restore | Restart rp2040_bridge; heartbeat resumes; AUTO re-enables. |

## T4 — Autonomous Command Timeout  [Mechanism #4]

| Field | Details |
| --- | --- |
| Prerequisites | Rover armed and following a mission. rp2040_bridge running normally. |
| Setup | Navigator actively publishing cmd_override at 10 Hz. |
| Action | Kill the navigator node only (keep rp2040_bridge alive). |
| Observe | Within 500 ms: serial log shows AUTO-TIMEOUT; PPM neutral. |
| Pass condition | Rover stops. Bridge + heartbeat continue. RP2040 does NOT enter EMERGENCY. |
| Restore | Restart navigator; re-arm via GQC START. |

## T5 — RF Link Timeout — Slave  [Mechanism #5]

| Field | Details |
| --- | --- |
| Prerequisites | Slave RP2040 running. RF link established. CH9 in slave (high) position. |
| Setup | Slave moving slowly in RF mode. |
| Action | Power off master RP2040 or block SX1278 antenna on master. |
| Observe | Slave serial: [RF_LINK_LOST] then MODE:EMERGENCY within 500 ms. |
| Pass condition | Slave stops within 500 ms. Master (RV1) is unaffected by this test. |
| Restore | Power on master RP2040; RF link recovers automatically. |

## T6 — SX1278 Power-Cycle Watchdog  [Mechanism #6]

| Field | Details |
| --- | --- |
| Prerequisites | Slave RP2040 powered on bench. RF link established. |
| Setup | Confirm [RF_LINK_OK] in serial log. |
| Action | Momentarily cut power to SX1278 module only (keep RP2040 powered). Wait 3 s. |
| Observe | Serial log: [RF_LINK_LOST] → [SX1278_REINIT opmode=0x…] → [RF_LINK_OK]. |
| Pass condition | RF link self-recovers without manual RP2040 reset. [SX1278_REINIT] line appears. |
| Restore | No action needed; system self-heals. |

## T7 — PPM Speed Limit  [Mechanism #7]

| Field | Details |
| --- | --- |
| Prerequisites | RP2040 connected. Oscilloscope or ppm_decoder RP2040 reading GP15 PPM output. |
| Setup | All modes can be tested; start with MANUAL. |
| Action | (a) Push throttle stick to 100% (MK32 shows 2000 µs). (b) Push spin stick to 100%. (c) Move an auxiliary channel (e.g. CH5) to 2000 µs. |
| Observe | (a)(b) Measured PPM pulse for CH1 and CH2 does not exceed 1650 µs. (c) CH5 (PPM idx 4) reaches 2000 µs freely — unaffected. |
| Pass condition | CH1 max = 1650 µs, CH2 max = 1650 µs. CH3–CH8 at full range. MK32 shows 2000 throughout. |
| Restore | Return sticks to neutral. |

## T8 — armed Topic Gate  [Mechanism #9]

| Field | Details |
| --- | --- |
| Prerequisites | Rover running, armed in AUTO mode, navigator publishing commands. |
| Setup | Rover in slow autonomous motion or bench-safe command stream. |
| Action | Publish: ros2 topic pub --once /rv1/armed std_msgs/Bool '{data: false}' |
| Observe | Navigator stops publishing cmd_override. Within 500 ms: AUTO-TIMEOUT on RP2040. GQC STATUS: ARM → MSL. |
| Pass condition | Rover stops. RP2040 shows AUTO-TIMEOUT (not EMERGENCY). Bridge still running. |
| Restore | Re-arm via GQC START button. |

## T9 — RTK Fix-Loss Safety  [Mechanism #11]

| Field | Details |
| --- | --- |
| Prerequisites | Rover running outdoors with RTK corrections. GQC connected and showing RTK FIX (green badge). |
| Setup | Rover armed in AUTO, RTK FIX confirmed. |
| Action | Disconnect RTCM3 corrections feed (unplug rtk_forwarder) or block sky view to force fix drop. In simulation: use simulator RTK quality controls. |
| Observe | Navigator logs fix-loss warning. Rover stops autonomous motion. Recording disabled. GQC RTK badge changes from RTK FIX to DGPS or 3D FIX. |
| Pass condition | Rover stops autonomy. RTK badge in GQC turns non-green within a few seconds. |
| Restore | Reconnect corrections; wait for RTK FIX before resuming. |

## T10 — Mission-Complete Auto-Disarm  [Mechanism #12]

| Field | Details |
| --- | --- |
| Prerequisites | Rover armed, short mission uploaded (3 waypoints minimum), GQC connected. |
| Setup | START pressed; rover following mission at 30% speed limit. |
| Action | Let the rover complete the full mission naturally — do not intervene. |
| Observe | At final waypoint: wp_active = −1 published. GQC STATUS: ARM → NA. WP counter shows complete. Rover stops. |
| Pass condition | Auto-disarm fires exactly once at true mission end (not at chunk boundaries). No manual action needed. |
| Restore | Upload new mission to resume. |

## T11 — E-STOP Button  [Mechanism #13]

| Field | Details |
| --- | --- |
| Prerequisites | Rover armed and moving in AUTO mode. GQC connected via WiFi. |
| Setup | Normal autonomous mission in progress. |
| Action | Tap the E-STOP button in GQC. |
| Observe | Three DISARM packets sent 100 ms apart (visible in monitor.py or Wireshark). Rover stops within 300 ms. STATUS → NA. |
| Pass condition | Rover stops. Both rovers receive DISARM (broadcast address). 3 packets confirmed in network capture. |
| Restore | Re-arm via START after re-uploading mission. |

## T12 — ARM Safety Gate  [Mechanism #15]

Three sub-tests, all using the GQC START button:

| Sub-test | Condition | Expected result |
| --- | --- | --- |
| T12a | No mission loaded on rover (STATUS = NA) | Toast message: "No mission on rover" — ARM blocked. |
| T12b | Mission loaded but rover GPS > 0.5 m from WP[0] | AlertDialog shows measured distance — ARM blocked. |
| T12c | Mission loaded, rover within 0.5 m of WP[0] | ARM proceeds normally; rover starts mission. |

## T13 — RC/HB Link-Mismatch Auto-Disarm  [Mechanism #16]

| Field | Details |
| --- | --- |
| Prerequisites | Both rovers connected. GQC running. |
| Setup | Move CH9 switch to slave position (> 1750 µs). RV2 powered off or disconnected. |
| Action | Wait > 2 s. |
| Observe | GQC checkLinkMismatch() detects switch = slave but no RV2 HEARTBEAT. DISARM × 3 sent automatically. |
| Pass condition | Auto-disarm fires within 2 s of mismatch detection. GQC logs the event. |
| Restore | Return CH9 to neutral/master position. Power on RV2. |

# 4.  Recommended Test Order for First Real-Rover Session

Complete all bench tests (no rover motion) before any outdoor drive tests. Each test leaves the system safe for the next one.

| Order | Test | Location | Rover motion |
| --- | --- | --- | --- |
| 1 | T7 — PPM speed limit | Bench (oscilloscope / ppm_decoder) | None |
| 2 | T2 — SWA emergency switch | Bench or static field | None |
| 3 | T1 — SBUS signal timeout | Bench or static field | None |
| 4 | T6 — SX1278 power-cycle watchdog | Bench | None |
| 5 | T11 — E-STOP button | Static field | None |
| 6 | T12 — ARM safety gate (a, b, c) | Static field | None |
| 7 | T5 — RF link timeout (slave) | Static field | Slow crawl |
| 8 | T8 — armed topic gate | Open area | Slow crawl |
| 9 | T4 — navigator kill (cmd timeout) | Open area | Slow crawl |
| 10 | T3 — bridge kill (HB timeout) | Open area | Slow crawl |
| 11 | T13 — link-mismatch auto-disarm | Open area | Optional |
| 12 | T10 — mission-complete auto-disarm | Open area | Full short mission |
| 13 | T9 — RTK fix-loss safety | Outdoors with RTK corrections | Full mission |

# 5.  Quick Reference — Safety Constants

| Constant | Value | Where defined | Controls |
| --- | --- | --- | --- |
| SBUS_TIMEOUT_MS | 200 ms | master/main.cpp | Max gap before SBUS loss → EMERGENCY |
| HB_TIMEOUT_MS | 300 ms | master + slave main.cpp | Max gap before Jetson HB loss blocks AUTO |
| CMD_TIMEOUT_MS | 500 ms | master + slave main.cpp | Max gap before autonomous command timeout |
| RF_TIMEOUT_MS | 500 ms | slave/main.cpp | Max gap before RF loss → EMERGENCY (slave) |
| SX_WATCHDOG_MS | 2000 ms | slave/main.cpp | SX1278 health check interval when RF lost |
| EMERGENCY_THRESH | 1700 µs | master + slave main.cpp | SWA (CH4) threshold for emergency trigger |
| SPEED_LIMIT_MAX | 1650 µs | master + slave main.cpp | Max throttle/spin hardware PPM (30% limit) |
| SPEED_LIMIT_MIN | 1350 µs | master + slave main.cpp | Min throttle/spin hardware PPM (30% limit) |
| ARM safety radius | 0.5 m | Android GQC — RoverPositionManager | Max distance from WP[0] to allow ARM |
| Mismatch timeout | 2 s | Android GQC — MainActivity | CH9 vs HEARTBEAT mismatch before auto-disarm |
| default_acceptance_radius | 0.1 m | rover1/2_params.yaml | Waypoint arrival radius in navigator |

AgriRover ROS2 System — Safety Documentation